KommuAssist2

EPS Torque Sensor Interceptor PCB

Hardware Design Study & Technical Documentation

| Field | Detail |
| --- | --- |
| Company | Kommu Sdn Bhd, Kota Damansara, Malaysia |
| Project | KommuAssist2 — Level 2 ADAS for Perodua vehicles |
| Document type | Hardware Design Study (OJT Project) |
| Drawn by | c.uaisyah02 |
| Supervisor/CTO | Ting |
| Schematic tool | EasyEDA (LCSC component library) |
| Document version | REV 1.0 |
| Date | July 2026 |
| Safety level | ASIL B (confirmed by CTO) |
| Status | Prototype / Study Phase |

This document covers the complete hardware design study for the EPS torque sensor interceptor PCB. It documents component selection rationale, simulation findings, schematic decisions, safety architecture, and gap analysis against the Safety-Rated EPS Torque Sensor Interface PCB Study Requirements.

# 1. Objective

This document records the complete hardware design study for a PCB that intercepts the torque sensor signal between the Perodua EPS ECU and its torque sensor, enabling KommuAssist2 to provide Level 2 ADAS steering assistance.

The PCB sits physically in the wire harness between the ECU and the torque sensor:

EPS ECU <──[Interceptor PCB]──> Torque Sensor

The default and safest state is OEM pass-through. Any fault, uncertainty, timeout, power issue, software error, signal mismatch, or diagnostic failure shall return the EPS ECU to the original torque sensor signal without delay.

NOTE: This document covers a study and prototype phase only. Public-road deployment requires functional safety assessment, legal review, vehicle-level validation, and regulatory approval per the requirements document.

# 2. System Overview

## 2.1 Core Concept

The interceptor PCB operates in two modes:

- OEM pass-through (default): The torque sensor signal passes directly through the PCB to the ECU unchanged. This is the hardware default — no firmware required.

- Boost active: The MCU modifies the torque signal within safe bounds when requested by KommuAssist2 via CAN bus, subject to supervisor approval.

## 2.2 Confirmed Signal Characteristics

| Parameter | Value | Source |
| --- | --- | --- |
| Sensor type | Analog (0–5V) | Wiring diagram from Ting |
| Signal channels | MAIN + SUB (dual complementary) | Wiring diagram |
| MAIN neutral voltage | ~2.5V at zero torque | Confirmed |
| MAIN signal range | 1.0V – 4.0V | Confirmed |
| MAIN + SUB relationship | Always sums to ~5V | Confirmed |
| ECU supply (VDD) | 5V — originates from ECU | Confirmed by Ting |
| Scanner value range | 0–255 (8-bit ADC inside ECU) | Confirmed |
| Boost threshold | Value > 200 (units TBC with Ting) | Pending confirmation |
| Fault code C1513 | Signal mismatch (MAIN+SUB ≠ 5V) | Known |
| Fault code C1515 | Signal stuck at 0 or 255 | Known |

## 2.3 Power Architecture

The PCB is powered from the ECU's 5V VDD pin — not from the 12V battery. This was confirmed by the CTO.

- Simpler design — no need for 12V step-down converter

- Cleaner power — already regulated by ECU

- PCB taps a small amount of ECU 5V while passing the full 5V through to the sensor unchanged

- 5V is regulated down to 3.3V on-board via AMS1117-3.3 LDO for all PCB chips

# 3. Safety Architecture

## 3.1 Concept C — Safety-Partitioned Interposer

The design implements Concept C from the requirements document — the most robust of the three recommended architectures:

- Control path: Main MCU (STM32G0B1CBT6) calculates boost values and drives DAC outputs

- Safety path: Independent supervisor MCU (STM32G030F6P6TR) independently monitors all signals and can force pass-through without relying on main MCU firmware

- Hardware enforcement: AND gate (SN74LVC1G08) ensures both supervisor signals must be HIGH simultaneously for boost to activate — cannot be bypassed by software

- Hardware default: TS5A23157 NC analog switch defaults to OEM pass-through with zero power — no firmware needed

## 3.2 Safety Goals Addressed

| Safety goal (from §5) | How addressed in design |
| --- | --- |
| Prevent unintended steering assist | Dual gate logic — AND gate requires both FORCE_PT and GATE_ENABLE HIGH |
| Prevent loss of driver steering authority | NC switch default — OEM signal always available hardware path |
| Prevent sustained wrong-direction assist | LM393 output monitor detects output mismatch → immediate pass-through |
| Prevent modification when vehicle state unknown | CAN timeout detection in supervisor firmware |
| Prevent modification during fault | Supervisor forces FORCE_PT=LOW on any fault condition |
| Immediate reversion on fault | Hardware AND gate — pull either input LOW → instant pass-through |
| No unsafe behavior during startup/reset | BOOT0 pull-down ensures clean boot; NC switch active during startup |
| No single fault causing sustained wrong output | Dual gate + independent supervisor + LM393 hardware monitor |
| Diagnostic fault logging | UART communication between MCU and supervisor; CAN reporting |

## 3.3 Non-Negotiable Design Principles Compliance

| Principle (§15) | Implementation |
| --- | --- |
| OEM pass-through is default state | TS5A23157 NC switch — physically connected by default |
| No software-only safety claim | Hardware AND gate + LM393 comparator — independent of all firmware |
| No CAN-command-only torque modification | CAN request → MCU calculates → Supervisor independently validates → AND gate |
| No active function without driver override | Boost only active when KommuAssist2 explicitly requests via CAN |
| No single fault causing sustained wrong output | Multiple independent layers: AND gate, supervisor, LM393 |

# 4. Component Selection Rationale

## 4.1 Main MCU — STM32G0B1CBT6 (LQFP-48)

| Parameter | Detail |
| --- | --- |
| Part number | STM32G0B1CBT6 |
| Package | LQFP-48 |
| Core | ARM Cortex-M0+ @ up to 64MHz |
| Flash | 128KB |
| RAM | 144KB |
| LCSC price | ~$2.22 single unit / ~$0.85 bulk (100+) |
| Key peripherals used | 2× DAC, 2× ADC, FDCAN, UART, SWD |

Why STM32G0B1CBT6:

- Dual DAC outputs (PA4 and PA5) — needed for independent MAIN and SUB signal generation

- FDCAN peripheral — required for vehicle CAN bus communication

- Sufficient ADC channels for reading MAIN_ADC and SUB_ADC

- 3.3V operation — matches system power rail

- LQFP-48 package — solderable by hand for prototype

- Good availability on LCSC

DAC strategy — Option B (firmware):

DAC_MAIN = boost_value

DAC_SUB  = MAX_VALUE - boost_value  // guarantees MAIN+SUB = 5V always

This ensures the complementary relationship is always maintained in firmware, preventing ECU fault C1513.

## 4.2 Safety Supervisor — STM32G030F6P6TR (TSSOP-20)

| Parameter | Detail |
| --- | --- |
| Part number | STM32G030F6P6TR |
| Package | TSSOP-20 |
| Core | ARM Cortex-M0+ @ up to 64MHz |
| Flash | 32KB |
| RAM | 8KB |
| LCSC price | ~$0.45 single unit |
| Key peripherals used | ADC (PA0, PA1, PA7), GPIO (HEARTBEAT, GATE_ENABLE, FORCE_PT, FAULT_OUT), UART |

Why STM32G030F6P6TR:

- Completely separate physical MCU from main MCU — cannot be overridden by main MCU firmware

- Has its own ADC to independently read MAIN_ADC and SUB_ADC

- Cheap (~$0.45) — cost-effective safety addition

- Same STM32 family as main MCU — familiar development environment

- TSSOP-20 package — small footprint

NOTE: TSSOP-20 package combines VDD+VDDA onto one physical pin (Pin 4) and VSS+VSSA onto one pin (Pin 5). This is correct per datasheet — not an error.

## 4.3 Op-amp — MCP6002DRG (SOP-8, Dual)

| Parameter | Detail |
| --- | --- |
| Part number | MCP6002DRG |
| Package | SOP-8 |
| Configuration | Dual op-amp (two channels in one package) |
| Supply voltage | 1.8V – 6.0V (single supply) |
| LCSC price | ~$0.20–0.40 |
| Used on | Sheet 2 (AFE buffer) + Sheet 5 (DAC scaling amplifier) |

Why MCP6002:

- Rail-to-rail output — can output very close to 0V and very close to VCC (5V), essential for full 0–5V signal range

- Single supply operation — works directly from VCC without negative rail

- Dual channel in one package — one chip handles both MAIN and SUB channels, reduces component count

- Low offset voltage — accurate enough for analog torque signal conditioning

- Used on two sheets — same chip in BOM simplifies ordering

Sheet 2 configuration (unity gain buffer):

- IN- tied to OUT — gain = 1 (buffer only)

- Purpose: high-impedance isolation of sensor signal — sensor doesn't know PCB is there

- Followed by RC filter (1kΩ + 100nF) and voltage divider (10kΩ/20kΩ) to scale 5V → 3.3V

Sheet 5 configuration (non-inverting amplifier, gain = 1.5):

- Rf = 10kΩ (feedback from OUT to IN-)

- Rg = 20kΩ (from IN- to GND)

- Gain = 1 + (Rf/Rg) = 1 + (10k/20k) = 1.5

- 3.3V × 1.5 = 4.95V ≈ 5V — scales MCU DAC output back to ECU range

NOTE: Op-amp on both Sheet 2 and Sheet 5 must be powered from VCC (5V), not +3.3V. LTSpice simulation confirmed that powering from 3.3V causes output clipping above 3.3V, making the upper half of the signal range inaccessible.

## 4.4 LDO Regulator — AMS1117-3.3 (SOT-223)

| Parameter | Detail |
| --- | --- |
| Part number | AMS1117-3.3 |
| Package | SOT-223 |
| Input voltage | 4.75V – 15V |
| Output voltage | 3.3V fixed |
| Max current | 1A |
| LCSC price | ~$0.10–0.20 |

Why AMS1117 (LDO) not a buck converter:

- Small voltage drop (5V → 3.3V, only 1.7V gap) — LDO is efficient enough

- Low current draw (~100–200mA total) — no need for high-efficiency switching

- No switching noise — buck converters generate high-frequency noise that would corrupt sensitive analog torque signals

- Simpler circuit — fewer components, easier to prototype

- Built-in thermal shutdown protection

## 4.5 CAN Transceiver — SN65HVD230DR-JSM (SOIC-8)

| Parameter | Detail |
| --- | --- |
| Part number | SN65HVD230DR-JSM |
| Package | SOIC-8 |
| Supply voltage | 3.3V |
| CAN standard | ISO 11898 |
| LCSC price | ~$0.30–0.50 |

Why SN65HVD230 over TJA1050:

- 3.3V operation — TJA1050 requires 5V supply which is not available for chips in this design

- RS pin to GND (via 10kΩ) = high-speed mode — suitable for automotive CAN

- 10kΩ pull-up on TX line (D pin) prevents floating when MCU is not driving

- 120Ω termination resistor between CANH and CANL — prevents signal reflections on CAN bus

## 4.6 Analog Switch — TS5A23157DGSR (MSOP-10)

| Parameter | Detail |
| --- | --- |
| Part number | TS5A23157DGSR |
| Package | MSOP-10 |
| Configuration | Dual SPDT analog switch |
| On-resistance | ~1Ω |
| Supply voltage | 2.7V – 4.3V |
| Signal bandwidth | > 100MHz |
| LCSC price | ~$0.20–0.30 |

Why TS5A23157:

- Dual SPDT — handles both MAIN and SUB channels simultaneously in one chip

- Normally Closed (NC) pin is physically connected when IN = LOW — hardware fail-safe guarantee

- On-resistance ~1Ω — negligible voltage drop, signal passes through cleanly

- Works on 3.3V supply — matches system rail

- Both channels controlled by same AND_OUT signal — MAIN and SUB always switch together, preventing C1513 fault

NOTE: NC behavior is the critical safety feature. When PCB has no power, IN pins are floating LOW, NC is connected, OEM signal passes to ECU. No firmware needed for safe state.

## 4.7 AND Gate — SN74LVC1G08DCKR (SOT-353)

| Parameter | Detail |
| --- | --- |
| Part number | SN74LVC1G08DCKR |
| Package | SOT-353 (5-pin) |
| Logic family | LVC (Low Voltage CMOS) |
| Supply voltage | 1.65V – 5.5V |
| Propagation delay | ~4ns |
| LCSC price | ~$0.05–0.10 |

Why hardware AND gate:

- Requirements document §15: No software-only safety claim

- If MCU firmware crashes, software AND logic dies with it

- Physical AND gate works even when all firmware is dead

- Two-key system: FORCE_PT AND GATE_ENABLE must both be HIGH — cannot be triggered by a single stuck-at-HIGH fault

- LVC family = 3.3V logic compatible

## 4.8 Output Comparator — LM393DR(LX) (SOIC-8, Dual)

| Parameter | Detail |
| --- | --- |
| Part number | LM393DR(LX) |
| Package | SOIC-8 |
| Configuration | Dual comparator |
| Output type | Open collector |
| Supply voltage | 2V – 36V |
| LCSC price | ~$0.05–0.10 |

Why LM393 with resistor averaging (not individual channel monitoring):

Original design monitored MAIN_OUT and SUB_OUT individually against 2.5V. This was found to be incorrect:

- During normal boost, MAIN can be anywhere from 1.0V to 4.0V

- Comparing MAIN individually against 2.5V would flag as fault when MAIN = 1.5V — even though that is perfectly normal

- The correct check is: MAIN + SUB = 5V (always, regardless of individual values)

Resistor averaging approach:

MAIN_OUT ── R26 (10kΩ) ──┐

├── average node → R31(1kΩ)+C24(100nF) → LM393 IN+

SUB_OUT  ── R30 (10kΩ) ──┘

VREF (2.5V) ──────────────────────────────────────────── LM393 IN-

Two equal resistors average MAIN and SUB. If MAIN+SUB=5V, average=2.5V always (regardless of individual values). Any fault that breaks the complementary relationship shifts the average away from 2.5V — comparator detects it.

RC filter (R31 1kΩ + C24 100nF) before comparator input:

- Automotive environment has electrical noise from motor switching, ignition, CAN traffic

- RC filter cutoff = 1/(2π×1k×100n) = ~1.6kHz

- Fast noise spikes filtered out → no false fault triggers

- Real faults (sustained voltage shift) still pass through after brief delay

Why VCC (5V) powers LM393 but pull-up resistors use +3.3V:

- LM393 VCC = 5V: must be powered above the signals it compares (0–5V signals)

- Pull-up to +3.3V: FAULT_OUT goes to supervisor GPIO which is 3.3V logic — 5V would damage it

## 4.9 Crystal Oscillator — 8MHz SMD (SMD-3225)

| Parameter | Detail |
| --- | --- |
| Frequency | 8MHz |
| Package | SMD 3225 (4-pin) |
| Load capacitors | 18pF × 2 (C15, C16) |
| Purpose | Accurate clock source for CAN bus timing |

Why external crystal:

- STM32G0B1 has internal HSI oscillator at 16MHz but accuracy is ±1% or worse

- CAN bus requires very precise timing — ±0.5% or better to avoid bit errors

- External 8MHz crystal + internal PLL = stable 64MHz system clock with automotive-grade accuracy

- 8MHz chosen as STM32CubeMX default — most example code assumes this frequency

# 5. LTSpice Simulation — MCP6002 AFE

## 5.1 Simulation Objective

LTSpice simulation was performed to verify the MCP6002 op-amp circuit behavior on Sheet 2 (AFE) and Sheet 5 (DAC scaling), specifically to validate signal range and identify clipping issues.

## 5.2 AFE Circuit (Sheet 2) — Unity Gain Buffer

Test setup:

- Input: 0–5V sinusoidal signal representing torque sensor MAIN output

- Op-amp: MCP6002 configured as unity gain buffer (IN- tied to OUT)

- Supply: VCC = 5V

- Load: RC filter (1kΩ + 100nF) + voltage divider (10kΩ/20kΩ)

Simulation findings:

- At VCC = 5V: Output correctly follows input across full 0–5V range ✅

- At VCC = 3.3V: Output clips at ~3.1V — upper portion of signal (3.1V–5V) is lost ❌

- This confirmed the critical finding: op-amp must be powered from VCC (5V), not +3.3V

Design decision based on simulation:

- MCP6002 VDD pin (Pin 8) connected to VCC (5V) on both Sheet 2 and Sheet 5

- This allows rail-to-rail output across full 0–5V torque signal range

- The 3.3V system rail is used only for digital chips (MCU, supervisor, AND gate)

## 5.3 DAC Scaling Circuit (Sheet 5) — Non-Inverting Amplifier

Test setup:

- Input: 0–3.3V sinusoidal signal representing MCU DAC output

- Op-amp: MCP6002 configured as non-inverting amplifier

- Gain resistors: Rf = 10kΩ, Rg = 20kΩ, theoretical gain = 1.5

- Supply: VCC = 5V

Simulation findings:

- Output range: 0V to ~4.95V (expected 5V — within component tolerance) ✅

- Gain verified: 3.3V × 1.5 = 4.95V — matches theoretical calculation ✅

- No clipping observed across full input range ✅

- Signal shape preserved — minimal distortion ✅

NOTE: LTSpice simulation screenshots are embedded in the EasyEDA schematic on Sheet 2 and Sheet 5 (Output Submodule) as visual reference.

# 6. Schematic Overview — All Sheets

| Sheet | Title | Status | Key components |
| --- | --- | --- | --- |
| Sheet 1 | Power Supply + CAN Interface + Debug | Complete | AMS1117-3.3, SN65HVD230, SWD headers |
| Sheet 2 (Output Submodule) | Output Submodule | Complete | MCP6002 ×2, TS5A23157, SN74LVC1G08, LM393 |
| AFE block | AFE & Conditioning | Complete | MCP6002 ×2, RC filters, voltage dividers |
| MCU block | Main MCU | Complete | STM32G0B1CBT6, 8MHz crystal |
| Supervisor block | Safety Supervisor | Complete | STM32G030F6P6TR |

## 6.1 Sheet 1 — Power Supply

Signal flow:

ECU 5V (VCC) → D1 Schottky (reverse polarity) → D2 TVS (load dump)

→ AMS1117-3.3 → +3.3V rail

→ C1 (10µF input cap) + C2 (22µF output cap)

→ R1 (330Ω) + LED2 (power indicator)

| Ref | Value | Package | Purpose |
| --- | --- | --- | --- |
| H1 | HDR-F-2.54 2×2 | Through-hole | ECU-side connector |
| H2 | HDR-F-2.54 2×2 | Through-hole | Sensor-side connector |
| U1 | AMS1117-3.3 | SOT-223 | 5V → 3.3V LDO regulator |
| C1 | 10µF | 0805 | LDO input decoupling |
| C2 | 22µF | 0805 | LDO output stability (mandatory per datasheet) |
| R1 | 330Ω | 0402 | LED current limiting |
| LED2 | LED | 0402 | Power indicator |
| D1 | SS14 Schottky | SOD-123 | Reverse polarity protection (added — Gap 1) |
| D2 | SMBJ5.0A TVS | SMB | Load dump protection (added — Gap 1) |

## 6.2 Sheet 2 (AFE) — Analog Front End & Conditioning

Signal flow per channel (×2 for MAIN and SUB):

Sensor signal (0-5V)

→ R_protect (100Ω) + D_clamp (3.6V zener) [short circuit + overvoltage protection]

→ R_bias (1MΩ to GND) [open wire detection]

→ MCP6002 unity gain buffer [high-Z isolation]

→ RC filter (R 1kΩ + C 100nF) [noise removal, cutoff ~1.6kHz]

→ Voltage divider (10kΩ/20kΩ) [5V → 3.3V scaling]

→ MAIN_ADC / SUB_ADC → MCU PA0/PA1 + Supervisor PA0/PA1

| Ref | Value | Purpose |
| --- | --- | --- |
| R_protect (×2) | 100Ω | Short circuit protection on MAIN and SUB lines (Gap 2) |
| D_clamp (×2) | BZX84C3V6 3.6V | Overvoltage clamp on MAIN and SUB (Gap 2) |
| R_bias (×2) | 1MΩ | Open wire pull-down (Gap 2) — 0V reading = broken wire |
| U2.1, U2.2 | MCP6002 | Unity gain buffer — high impedance sensor isolation |
| R2, R7 | 1kΩ | RC filter resistor per channel |
| C4, C5 | 100nF | RC filter capacitor per channel |
| R3, R6 | 10kΩ | Voltage divider top resistor |
| R4, R5 | 20kΩ | Voltage divider bottom resistor |

## 6.3 MCU Block — STM32G0B1CBT6

| Pin | Net label | Direction | Purpose |
| --- | --- | --- | --- |
| PA0 (Pin 11) | MAIN_ADC | Input | Read conditioned torque MAIN signal |
| PA1 (Pin 12) | SUB_ADC | Input | Read conditioned torque SUB signal |
| PA4 (Pin 15) | DAC_MAIN | Output | DAC channel 1 — MAIN modified signal |
| PA5 (Pin 16) | DAC_SUB | Output | DAC channel 2 — SUB modified signal |
| PB8 (Pin 47) | CAN_RX | Input | FDCAN receive from vehicle bus |
| PB9 (Pin 48) | CAN_TX | Output | FDCAN transmit to vehicle bus |
| PC6 (Pin 30) | HEARTBEAT_OUT | Output | 50ms pulse to supervisor watchdog |
| PA9 (Pin 29) | UART_TX | Output | Diagnostic UART to supervisor |
| PA8 (Pin 28) | UART_RX | Input | Diagnostic UART from supervisor |
| PA13 (Pin 35) | MCU_SWDIO | Debug | SWD programming data |
| PA14 (Pin 36) | MCU_SWDCLK | Debug | SWD programming clock + BOOT0 |
| PC14/PC15 (Pin 2/3) | Crystal X1 | Clock | 8MHz external crystal |
| VBAT (Pin 4) | +3.3V | Power | Backup domain supply |
| VREF+ (Pin 5) | +3.3V + 100nF | Power | ADC reference voltage |
| VDD/VDDA (Pin 6) | +3.3V | Power | Main and analog supply |

## 6.4 Safety Supervisor Block — STM32G030F6P6TR

| Pin | Net label | Direction | Purpose |
| --- | --- | --- | --- |
| PA0 (Pin 7) | MAIN_ADC | Input | Independent torque MAIN read |
| PA1 (Pin 8) | SUB_ADC | Input | Independent torque SUB read |
| PA2 (Pin 9) | HEARTBEAT_IN | Input | MCU watchdog heartbeat |
| PA3 (Pin 10) | UART_TX | Output | Diagnostic to MCU (crossover) |
| PA4 (Pin 11) | UART_RX | Input | Diagnostic from MCU (crossover) |
| PA5 (Pin 12) | GATE_ENABLE | Output | Arms output stage (to AND gate) |
| PA6 (Pin 13) | FORCE_PT | Output | Active safety signal (to AND gate) |
| PA7 (Pin 14) | 5V_MON | Input | ECU 5V rail health monitoring |
| PB0 (Pin 15) | FAULT_OUT | Input | Hardware output fault from LM393 |
| PA13 (Pin 18) | SWDIO | Debug | SWD programming data |
| PA14 (Pin 19) | SWDCLK | Debug | SWD programming clock + BOOT0 |
| VDDA/DDA (Pin 4) | +3.3V | Power | Combined VDD+VDDA (TSSOP-20) |

## 6.5 Sheet 2 — Output Submodule

Sub-circuit 1: DAC Scaling (MCP6002 U6.1 + U6.2)

DAC_MAIN (0-3.3V from MCU PA4)

→ R22 (1kΩ) + C19 (100nF) [RC filter, removes DAC glitches]

→ U6.1 MCP6002 IN+ (Pin 3)

→ Gain = 1 + (R19/R20) = 1 + (10k/20k) = 1.5

→ DAC_MAIN_OUT (0-5V) → TS5A23157 NO1

DAC_SUB (0-3.3V from MCU PA5) — identical circuit

→ DAC_SUB_OUT (0-5V) → TS5A23157 NO2

Sub-circuit 2: Dual Gate Logic (SN74LVC1G08 U9)

FORCE_PT (supervisor PA6) → AND gate Pin A

GATE_ENABLE (supervisor PA5) → AND gate Pin B

→ AND_OUT → TS5A23157 IN1 + IN2

AND_OUT = HIGH only when BOTH FORCE_PT=HIGH AND GATE_ENABLE=HIGH

Sub-circuit 3: Analog Switch (TS5A23157 U7)

AND_OUT = LOW (default):

COM1(Pin10)←→NC1(Pin9): MAIN(OEM) → MAIN_OUT → ECU

COM2(Pin6) ←→NC2(Pin7): SUB(OEM)  → SUB_OUT  → ECU

AND_OUT = HIGH (boost active):

COM1(Pin10)←→NO1(Pin2): DAC_MAIN_OUT → MAIN_OUT → ECU

COM2(Pin6) ←→NO2(Pin4): DAC_SUB_OUT  → SUB_OUT  → ECU

Sub-circuit 4: Output Monitor (LM393 U8)

MAIN_OUT ── R26 (10kΩ) ──┐

├── R31 (1kΩ) ──┬── LM393 Pin3 (1IN+)

SUB_OUT  ── R30 (10kΩ) ──┘               └── C24 (100nF) → GND

VREF (2.5V) ─────────────────────────────── LM393 Pin2 (1IN-)

If (MAIN+SUB)/2 ≠ 2.5V → FAULT_OUT goes LOW → Supervisor PB0

# 7. Gap Analysis Against Requirements Document

## 7.1 Gaps Identified and Addressed

| Gap | Requirement ref | Resolution | Sheet |
| --- | --- | --- | --- |
| Reverse polarity protection | §7.6 | Added SS14 Schottky diode (D1) | Sheet 1 |
| Load dump protection | §7.6 | Added SMBJ5.0A TVS diode (D2) | Sheet 1 |
| Short circuit protection on signal lines | §7.3 | Added 100Ω series resistors on MAIN and SUB | AFE |
| Overvoltage clamp on signal lines | §7.3 | Added BZX84C3V6 3.6V zener on MAIN and SUB | AFE |
| Open wire detection | §7.3 | Added 1MΩ pull-down on MAIN and SUB lines | AFE |
| ESD protection on CAN connector | §7.7 | Added PESD1CAN on CANH and CANL at H3 | Sheet 6 CAN |
| Output monitor false-triggering | §7.2 | Replaced individual channel monitoring with resistor averaging circuit | Output Submodule |
| Noise on output monitor | §7.3 | Added RC filter (1kΩ + 100nF) before comparator input | Output Submodule |

## 7.2 Known Remaining Gaps (Accepted for Prototype Stage)

| Gap | Requirement ref | Status | Mitigation |
| --- | --- | --- | --- |
| AEC-Q component qualification | §7.1 | Partially addressed | Industrial grade components used; automotive variants to be evaluated for production |
| Separate sensor VDD monitoring | §7.2 | Accepted simplification | 5V_MON covers same rail; dedicated monitoring in V2 |
| ESD protection on H1/H2 signal connectors | §7.7 | Partial | PESD1CAN added to CAN; H1/H2 ESD arrays to be added in V2 |
| Cranking/brownout behavior | §7.6 | Partial | Supervisor 5V_MON detects undervoltage; AMS1117 handles brownout |
| Full FMEDA/FTA analysis | §13 | Not yet | Required before moving from prototype to production |
| Cybersecurity threat analysis | §10 | Not yet | Required before any vehicle deployment |
| Formal HARA | §4 | Not yet | Required before any vehicle deployment |

## 7.3 Strong Alignment Points

| Requirement | Section | Implementation |
| --- | --- | --- |
| Concept C Safety-Partitioned Interposer | §6 | Separate control MCU + safety supervisor MCU + hardware AND gate |
| Independent safety supervisor | §7.2 | STM32G030 separate physical MCU with own ADC, firmware, and GPIO |
| Hardware pass-through guarantee | §7.4 | TS5A23157 NC switch — physically connected with zero power, zero firmware |
| No software-only safety claim | §15 | Hardware AND gate + LM393 comparator — independent of all firmware |
| Dual channel correlation | §7.3 | MAIN+SUB sum check in both MCU firmware and LM393 hardware |
| Output stage fault detection | §7.3 | LM393 resistor averaging circuit monitors MAIN_OUT + SUB_OUT relationship |
| CAN safety gating | §7.5 | CAN request → MCU → Supervisor validates independently → AND gate controls switch |
| Heartbeat watchdog | §7.2 | MCU sends 50ms pulse; supervisor forces pass-through if pulse stops |
| Power rail monitoring | §7.2 | Supervisor reads 5V_MON on PA7 via voltage divider |
| OEM pass-through is default | §15 | Hardwired via TS5A23157 NC default state |

# 8. Open Questions for CTO Review

| # | Question | Why it matters | Impact if not answered |
| --- | --- | --- | --- |
| 1 | Is boost threshold '200' in scanner units (0–255) or raw ADC value? | Determines firmware threshold logic | Wrong threshold → boost activates at wrong torque |
| 2 | What exactly is the TS pin on the torque sensor connector? | May need extra connector pin and schematic changes | Potential missed signal — unknown behavior |
| 3 | How much current can safely be drawn from ECU 5V VDD? | Verify power budget safe for PCB | Could disturb sensor supply or trigger ECU fault |
| 4 | Should signal correlation check also be in hardware comparator, or is software sufficient? | Sheet 2 design decision | Potential gap in §7.3 analog path coverage |

# 9. Net Label Cross-Reference

| Net label | From | To | Purpose |
| --- | --- | --- | --- |
| VCC | H1 connector | AMS1117, voltage dividers, op-amps | ECU 5V supply |
| +3.3V | AMS1117 output | All digital chips | Regulated 3.3V |
| MAIN | H2 sensor connector | AFE input, TS5A23157 COM1 | OEM MAIN torque signal |
| SUB | H2 sensor connector | AFE input, TS5A23157 COM2 | OEM SUB torque signal |
| MAIN_ADC | AFE voltage divider | MCU PA0, Supervisor PA0 | Scaled MAIN (0-3.3V) |
| SUB_ADC | AFE voltage divider | MCU PA1, Supervisor PA1 | Scaled SUB (0-3.3V) |
| DAC_MAIN | MCU PA4 | Sheet 5 op-amp U6.1 | Modified MAIN (0-3.3V) |
| DAC_SUB | MCU PA5 | Sheet 5 op-amp U6.2 | Modified SUB (0-3.3V) |
| DAC_MAIN_OUT | Op-amp U6.1 output | TS5A23157 NO1 | Scaled modified MAIN (0-5V) |
| DAC_SUB_OUT | Op-amp U6.2 output | TS5A23157 NO2 | Scaled modified SUB (0-5V) |
| AND_OUT | SN74LVC1G08 output | TS5A23157 IN1, IN2 | Combined supervisor approval |
| FORCE_PT | Supervisor PA6 | AND gate Pin A | Active safety signal |
| GATE_ENABLE | Supervisor PA5 | AND gate Pin B | Output stage arm signal |
| MAIN_OUT | TS5A23157 NC1/NO1 | ECU via H1, LM393 IN+ | Final MAIN signal to ECU |
| SUB_OUT | TS5A23157 NC2/NO2 | ECU via H1, LM393 IN+ | Final SUB signal to ECU |
| FAULT_OUT | LM393 1OUT (Pin 1) | Supervisor PB0 (Pin 15) | Hardware output fault signal |
| VREF | R27+R28 divider | LM393 Pin 2 (1IN-) | 2.5V comparator reference |
| 5V_MON | R13+R14 divider | Supervisor PA7 | ECU 5V health monitor (reads 2.5V nominal) |
| HEARTBEAT_OUT | MCU PC6 | Supervisor PA2 | 50ms watchdog pulse |
| UART_TX | MCU PA9 / Supervisor PA3 | Cross-connected | UART diagnostic (TX of one = RX of other) |
| UART_RX | MCU PA8 / Supervisor PA4 | Cross-connected | UART diagnostic (RX of one = TX of other) |
| CAN_TX | MCU PB9 | SN65HVD230 Pin 1 | CAN transmit |
| CAN_RX | MCU PB8 | SN65HVD230 Pin 4 | CAN receive |
| MCU_SWDIO | MCU PA13 | MCU_HEADER1 Pin 1 | Main MCU SWD data |
| MCU_SWDCLK | MCU PA14 | MCU_HEADER1 Pin 2 | Main MCU SWD clock |
| SWDIO | Supervisor PA13 | SUPERVISOR_HEADER1 Pin 1 | Supervisor SWD data |
| SWDCLK | Supervisor PA14 | SUPERVISOR_HEADER1 Pin 2 | Supervisor SWD clock |

# 10. Complete Bill of Materials

| Ref | Component | Value/Part | Package | Qty | LCSC est. price |
| --- | --- | --- | --- | --- | --- |
| U1 | LDO Regulator | AMS1117-3.3 | SOT-223 | 1 | $0.10–0.20 |
| U2 | Op-amp (AFE) | MCP6002DRG | SOP-8 | 1 | $0.20–0.40 |
| U3 | Main MCU | STM32G0B1CBT6 | LQFP-48 | 1 | $2.22 |
| U4 | CAN Transceiver | SN65HVD230DR | SOIC-8 | 1 | $0.30–0.50 |
| U5 | Safety Supervisor | STM32G030F6P6TR | TSSOP-20 | 1 | $0.45 |
| U6 | Op-amp (Output) | MCP6002DRG | SOP-8 | 1 | $0.20–0.40 |
| U7 | Analog Switch | TS5A23157DGSR | MSOP-10 | 1 | $0.20–0.30 |
| U8 | Comparator | LM393DR(LX) | SOIC-8 | 1 | $0.05–0.10 |
| U9 | AND Gate | SN74LVC1G08DCKR | SOT-353 | 1 | $0.05–0.10 |
| X1 | Crystal | 8MHz SMD | SMD-3225 | 1 | $0.10–0.20 |
| D1 | Schottky Diode | SS14 | SOD-123 | 1 | $0.05 |
| D2 | TVS Diode | SMBJ5.0A | SMB | 1 | $0.10 |
| D_clamp ×2 | Zener Diode | BZX84C3V6 | SOT-23 | 2 | $0.05 each |
| D_ESD_CAN | CAN ESD | PESD1CAN | SOT-23 | 1 | $0.15 |
| LED2 | LED | Any green | 0402 | 1 | $0.02 |
| H1, H2 | Connector | HDR-F-2.54 2×2 | Through-hole | 2 | $0.10 each |
| H3 | Connector | HDR-F-2.54 1×2 | Through-hole | 1 | $0.05 |
| MCU_HEADER1, SUP_HEADER1 | SWD Header | HDR-M-2.54 1×3 | Through-hole | 2 | $0.05 each |
| Capacitors (all) | Various | 100nF, 1µF, 4.7µF, 10µF, 18pF, 22µF | 0402/0805 | ~20 | ~$0.30 total |
| Resistors (all) | Various | 100Ω, 330Ω, 1kΩ, 10kΩ, 20kΩ, 1MΩ | 0402 | ~30 | ~$0.30 total |

Estimated total BOM cost (single prototype unit): ~$5.50 – $7.00

# 11. Next Steps

## 11.1 Immediate Actions (Before PCB Layout)

- Get answers from CTO on all 4 open questions in Section 8

- Add protection components to schematic (D1, D2, D_clamp, R_protect, R_bias, PESD1CAN)

- Run ERC (Electrical Rules Check) in EasyEDA — fix all errors

- Final schematic review with CTO

## 11.2 PCB Layout Phase

- Analog/digital partitioning — keep analog signal traces away from digital switching

- Place decoupling caps as close as possible to each chip's power pin

- CAN bus layout rules — controlled impedance, matched length CANH/CANL

- Ground plane strategy — single solid ground plane

- Keep MAIN and SUB analog traces away from CAN bus traces

- Test points on all key nets for bench debugging

## 11.3 Validation Stages (Per Requirements Document §12)

| Stage | Activity | Gate criteria |
| --- | --- | --- |
| Stage 1 | Bench signal characterization | Measure actual Perodua torque sensor signals |
| Stage 2 | Signal emulator bench test | Verify pass-through, no-glitch switching, fault detection |
| Stage 3 | Hardware-in-the-loop | Simulate CAN, sensor faults, power faults |
| Stage 4 | Vehicle static test (wheels lifted) | Verify EPS behavior, fail-safe, power cycle |
| Stage 5 | Closed-track test | Only after independent safety review |

NOTE: No public-road testing shall occur during development. No deployment without independent safety assessment. (Requirements document §15)

# 12. Document Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| REV 1.0 | July 2026 | c.uaisyah02 | Initial release — complete hardware design study documentation |